Project Flow & System Design Document

| Prepared by | - Sumith Dhage - Vedant Mohite - Yash Potdar |
| --- | --- |
| Project domain | Automotive simulation and vehicle dynamics |
| Document purpose | Python Megaproject Documentation |

Project Overview

The Car Emulator GUI & Vehicle Motion Simulator is a Python desktop application that reproduces core longitudinal vehicle operations - engine start, acceleration, braking and stop - through a responsive graphical interface. It combines an Excel-driven parameter layer, a deterministic physics engine, and a continuously updated velocity-time graph.

Project Objective

- Develop a realistic vehicle motion simulator using Python and CustomTkinter.

- Model longitudinal vehicle dynamics using real automotive physics equations.

- Implement an Excel-driven architecture where vehicle parameters and torque maps can be modified without changing code.

- Simulate key vehicle operations such as Start, Accelerate, Brake, and Stop through an interactive GUI.

- Visualize vehicle performance through a live-updating Velocity-Time (v-t) graph using Matplotlib.

System Architecture:

What Data the System Uses:

| Data Group | Example Fields |
| --- | --- |
| Vehicle Constants | Vehicle Mass (M_v), Equivalent Mass (M_e), Drag Coefficient (Cd), Frontal Area (Af), Rolling Resistance Coefficient (f_r), Tire Radius, Air Density (ρ), Gravity (g) |
| Powertrain | Engine RPM, Throttle Position (%), Torque Output (Nm), Torque Map Lookup Values |
| Vehicle Motion States | Vehicle Speed (km/h), Vehicle Speed (m/s), Acceleration (m/s²), Simulation Time (s) |
| Force Calculations | Traction Force (Ft), Aerodynamic Drag Force (Fa), Rolling Resistance Force (Fr), Brake Force (F_brake), Net Force |
| Driver Control | Start Engine Command, Accelerate Command, Brake Command, Stop Engine Command, Throttle Level (%) |
| Graph & Telemetry Data | Time History Array, Velocity History Array, Live Speed Display, Engine RPM Display, Force Readout Display |

Expected Analytical Output:

- Engine ON/OFF state monitoring

- Vehicle speed monitoring in km/h

- Real-time Velocity-Time (v-t) graph generation

- Engine ON/OFF state monitoring

- Time-history data collection for graph plotting

- Simulation runtime tracking

Python Technologies and Their Roles:

Python

- Controls simulation logic, GUI functions, and vehicle dynamics calculations.

CustomTkinter

- Creates the interactive GUI, buttons, and telemetry displays.

Pandas

- Loads vehicle constants and torque map data from Excel files.

NumPy

- Performs numerical and mathematical calculations efficiently.

Matplotlib

- Generates the real-time Velocity-Time (v-t) graph.

OpenPyXL

- Supports reading and writing Excel-based vehicle data.

Vehicle Physics Engine

- Calculates traction, drag, rolling resistance, braking force, and acceleration.

Flow Diagram:

Flow Diagram Explanation:

1. Vehicle Dynamics

- Initializes and stores the main vehicle parameters such as mass, equivalent mass, speed, throttle input, and brake status.

- Acts as the core simulation module for vehicle motion calculations.

2. 2-D Torque Table

- Uses engine RPM and throttle position to obtain torque from the Excel-based Torque Map.

- Converts engine torque into traction force (Ft) that drives the vehicle forward.

3. Resistance Factor

- Calculates the opposing forces acting on the vehicle.

- Includes aerodynamic drag (Fa), rolling resistance (Fr), and braking force (F_brake).

4. Acceleration

- Computes vehicle acceleration using the net force acting on the vehicle.

- Updates vehicle speed every 100 ms using Euler integration.

5. Line Graph

- Continuously plots Vehicle Speed versus Time.

- Provides real-time visualization of acceleration and braking performance.

Overall Working

- Vehicle parameters are processed through the torque lookup and force calculation stages.

- Net acceleration is determined and vehicle speed is updated.

- The updated speed is displayed on the live Velocity-Time graph, and the process repeats until the simulation is stopped.

Analysis Logic:

1. Vehicle Parameter Initialization

- Load vehicle constants and torque map data from the Excel workbook.

- Initialize speed, throttle position, engine RPM, and simulation variables.

2. RPM Calculation

- Convert vehicle speed from km/h to m/s.

- Calculate engine RPM using tire radius and gear ratio relationships.

3. Torque Lookup Analysis

- Use the calculated RPM and current throttle position to access the 2-D Torque Map.

- Retrieve the corresponding engine torque value.

4. Traction Force Calculation

- Convert engine torque into traction force acting at the wheels.

- Determine the driving force available for vehicle motion.

5. Resistance Force Analysis

- Calculate aerodynamic drag force.

- Calculate rolling resistance force.

- Apply brake when the brake function is active.

- Determine the total resistive force acting against vehicle motion.

6. Acceleration Analysis

- Compute the net force acting on the vehicle.

- Calculate acceleration using Newton's Second Law and the equivalent vehicle mass.

7. Graphical Analysis

- Store current time and speed values.

- Plot a live Velocity-Time (v-t) graph to visualize vehicle performance.

Illustrative Decision Rules :

| Condition | Example Rule | Result |
| --- | --- | --- |
| Vehicle Acceleration | Throttle position increases and traction force exceeds resistance forces | Vehicle speed increases |
| Vehicle Braking | Brake button is active and brake force is applied | Vehicle speed decreases |
| Engine Start | Start Engine button is pressed | Simulation loop begins |
| Engine Stop | Stop Engine button is pressed | Vehicle states and graphs are reset |
| High Resistance Load | Aerodynamic drag and rolling resistance increase with speed | Acceleration reduces |
| Torque Generation | RPM and throttle position match a torque map entry | Traction force is generated |
| Maximum Throttle | Throttle reaches the limit of 85% | Maximum available torque is applied |
| Vehicle at Rest | Speed is near 0 km/h and no throttle is applied | Vehicle remains stationary |
| Live Graph Update | New speed value is calculated | Velocity-Time graph is refreshed |
| Continuous Simulation | Engine remains ON | Physics calculations repeat in a loop |

Dashboard and Report Structure:

User Inputs

- Click START ENGINE to begin the simulation.

- Use ACCELERATE to increase the throttle position.

- Use BRAKE to apply vehicle braking and reduce speed.

- Monitor live vehicle parameters and graph updates.

- Click STOP ENGINE to end and reset the simulation.

Example Final Summary

- Simulation Time: 60 seconds

- Maximum Vehicle Speed: 78 km/h

- Average Vehicle Speed: 42 km/h

- Maximum Engine RPM: 3800 RPM

- Peak Traction Force: 4200 N

- Total Brake Applications: 5

- Velocity-Time Graph Generated: Yes

- Engine Status: Successfully Operated

- Vehicle Response: Stable Acceleration and Deceleration Achieved

- Final Vehicle Speed: 0 km/h (After Stop Engine)

- Overall Simulation Status: Vehicle Dynamics Successfully Simulated

Report Outputs

- Vehicle speed profile over time

- Engine RPM variation over time

- Force analysis (Ft, Fa, Fr, F_brake)

- Acceleration and deceleration performance

- Velocity-Time graph

- Vehicle dynamics performance summary

- Simulation statistics and operational results

Final System Architecture:

Vehicle Data Excel File (Vehicle Constants + Torque Map)
↓
Data Loading & Validation (Pandas/OpenPyXL)
↓
Vehicle Parameter Initialization
↓
RPM & Torque Map Lookup Analysis
↓
Traction Force and Resistance Force Calculation
↓
Acceleration & Vehicle Dynamics Computation
↓
Speed Update using Euler Integration
↓
Real-Time Telemetry Display & Velocity-Time Graph
↓
Vehicle Performance Analysis and Simulation Results

Python Syllabus Coverage:

| Syllabus | Use in Project |
| --- | --- |
| Fundamentals | Variables, constants, operators, input/output, and state variables |
| Decision Making | Engine ON/OFF states, throttle control, braking logic, force evaluation |
| Loops | Continuous 100 ms simulation update cycle |
| Functions | Separate functions for RPM calculation, torque lookup, force calculation, plotting, and simulation control |
| Collections | Lists and arrays for storing time-history and velocity-history data |
| File Handling | Reading vehicle parameters and torque maps from Excel files |
| Exception Handling | Handling missing Excel files, invalid data, and runtime errors |
| OOP | Classes such as Vehicle Simulator, Physics Engine, ExcelDataLoader, and Graph Manager |
| Modules/Packages | CustomTkinter, Pandas, NumPy, Matplotlib, OpenPyXL, Math |

Project Deliverables

- Python source code for the Vehicle Motion Simulator

- Vehicle data Excel workbook (vehicle_data.xlsx)

- CustomTkinter-based graphical user interface

- Real-time Velocity-Time (v-t) graph visualization

- Vehicle dynamics and force analysis results

- Project documentation and presentation

Key Point

- Excel Data = Provides vehicle constants and torque map information.

- Pandas/OpenPyXL = Loads and processes vehicle data.

- Physics Engine = Calculates RPM, torque, forces, acceleration, and speed.

- Vehicle Dynamics Model = Simulates real-world vehicle motion.

- Matplotlib Graph = Visualizes vehicle speed over time.

- CustomTkinter GUI = Presents telemetry, controls, and simulation results.